Course: CSE 204 – Computer Networks LAB 5 – DHCP and Routing Due date: November 22nd, 2025

| Student name | Phan Ngoc Hanh Nhi |
| --- | --- |
| Student ID | 2131209002 |

# Goal

- Students know how to configure DHCP on router.

- Students know how to connect 2 router using Serial Interface

- Students know how to configure Static routing

- Students know how to configure Dynamic routing (RIPv2)

# Assignment list

Note: In the first time we open the Router CLI, if the initial configuration dialog appears, please enter “no”.

II.1 DHCP Configuration

- Open Cisco Packet Tracer and implement the network as shown below:

- On the Router:

Router>enable

Router#config terminal

| Router(config)#ip dhcp pool net172 | //enter dhcp config mode |
| --- | --- |
| Router(dhcp-config)#network 172.10.0.0 255.255.0.0 | //dhcp for network 172.10.0.0/16 |
| Router(dhcp-config)#default-router 172.10.0.1 | //default gateway for PCs client |
| Router(dhcp-config)#exit |  |
| Router(config)#ip dhcp excluded-address 172.10.0.1 | //do not assign 172.10.0.1 to hosts |
| Router(config)#interface fastEthernet 0/0 |  |
| Router(config-if)#ip address 172.10.0.1 255.255.0.0 | //set ip for interface fa0/0 on Router |
| Router(config-if)#no shutdown |  |

- On the PCs

On PCs, set DHCP on.

Result:

II.2 Connect 2 routers using serial interface

- Add serial interface module for router

- Connect 2 routers by serial cable

- Configure connection between 2 routers

- On Router R1

Router>enable

R1#config terminal

Router(config)#hostname R1

R1(config)#interface serial1/0

R1(config-if)#ip address 10.0.0.1 255.255.255.252

R1(config-if)#no shutdown

- On Router R2

Router>enable

R2#config terminal

Router(config)#hostname R2

R2(config)#interface serial1/0

R2(config-if)#clock rate 128000

R2(config-if)#ip address 10.0.0.2 255.255.255.252

R2(config-if)#no shutdown

- Ping 2 Routers and show the result:

II.3 Static Routing

- Configure DHCP and assign IP to all Router interfaces

- On Router R1

      - R1 (config)#ip route 172.16.0.0 255.255.0.0 10.0.0.3

      - OR: R1 (config)#ip route 172.16.0.0 255.255.0.0 se1/0

- On Router R2

      - R2 (config)#ip route 192.168.1.0 255.255.255.0 10.0.0.2

      - OR: R2 (config)#ip route 192.168.1.0 255.255.255.0 se1/0

- Show routing table

      - Router#show ip route

- Result (ping between PCs):

II.4 Dynamic Routing – RIPv2

- On R1

Router(config)#router rip

Router(config-router)#version 2

Router(config-router)#network 192.168.1.0

Router(config-router)#network 10.0.0.0

Router(config-router)#network 10.10.0.0

- On R2

Repeat those steps on R1

- On R3

Repeat those steps on R1

- Results:

III. Extra Assignments

III.1 DHCP & Static Routing

- Configure DHCP and Static Routing using the model below

      - Show all configuration commands at each step for each device

      - Show the routing table on each router and ping results between PCs

- Configure DHCP and Static Routing using the model below

      - Show all configuration commands at each step for each device

      - Show the routing table on each router and ping results between PCs

III.2 Dynamic Routing

- Configure DHCP and Dynamic Routing using the model below

      - Show all configuration commands at each step for each device

      - Show the routing table on each router and ping results between PCs

- Configure DHCP and Dynamic Routing using the model below

      - Show all configuration commands at each step for each device

      - Show the routing table on each router and ping results between PCs