Course: CSE 204 – Computer Networks LAB 6 – ACL & IPv6 & NAT & FileZilla & Wireshark Due date: November 29th, 2025

| Student name | Tran Mai Anh |
| --- | --- |
| Student ID | 2231200062 |

# Goal

- Implementing Access Control List

- Implementing Static NAT, Dynamic NAT and PAT

- Implementing IPv6 Routing

- Implementing File Transfer Protocol using FileZilla

- Using Wireshark software to monitor the network traffic

# Assignment list

II.1. Access Control List (ACL)

Given a network as shown in the figure below.

- Config IP address to all Routers’ interfaces and PCs. Show the results of each step:

- Configure Routing protocols on 2 routers, so that all hosts and servers can ping to each other. Result:

- Configure Access Control List on the Router R1 to ensure: LAN host CANNOT connect to other devices in the network.

- Result:

II.2. IPv6 Routing

Given a network as shown in the figure below.

- On Router R1:

Router>enable
Router#config terminal
Router(config)#ipv6 unicast-routing
Router(config)#interface fastEthernet0/0
Router(config-if)#ipv6 address 1::1/64
Router(config-if)#no shut
Router(config-if)#exit
Router(config)#interface fastEthernet0/1
Router(config-if)#ipv6 address 2::1/64
Router(config-if)#no shut

Router(config-if)#exit

Router(config)#ipv6 route 3::/64 2::2

- On Router R2:

Router>enable

Router#config terminal

Router(config)#ipv6 unicast-routing

Router(config)#interface fastEthernet0/0

Router(config-if)#ipv6 address 3::1/64

Router(config-if)#no shut

Router(config-if)#exit

Router(config)#interface fastEthernet0/1

Router(config-if)#ipv6 address 2::2/64

Router(config-if)#no shut

Router(config-if)#exit

Router(config)#ipv6 route 1::/64 2::1

- Ping 2 PCs and show the result:

II.3. NAT

- Static NAT

Given a network as shown in the figure below.

- Config IP address to Routers’ interfaces and PCs. Show the results of each step:

- Assume that Router R1 can use the public IP address range 199.99.9.0/24. Configure Static NAT on Router R1 to translate IP address of Mail Server to public IP address.

- Configure static routing on router ISP to the NAT address:

ISP(config)#ip route 199.99.9.0 255.255.255.0 200.20.10.2

- Configure default route on router R1:

R1(config)#ip route 0.0.0.0 0.0.0.0 200.20.10.3

- Can Mail Server ping to Web Server? Result:

Yes, Mail Server can ping to Web Server

- Check the IP NAT translation.

R1#show ip nat translations

- Result:

- Dynamic NAT

Given a network as shown in the figure below.

- Config IP address to Routers’ interfaces and PCs. Show the results of each step:

- Assume that Router R1 can use the IP address range my-pool (199.99.9.40 to 199.99.9.62/27). Configure Dynamic NAT on Router R1 to translate local IP address of Mail Server to my-pool.

- Configure static routing on router ISP to the NAT address:

ISP(config)#ip route 199.99.9.32 255.255.255.224 200.20.10.2

- Configure default route on router R1:

R1(config)#ip route 0.0.0.0 0.0.0.0 200.20.10.3

- Can Mail Server ping to Web Server? Result:

Yes, Mail Server can ping to Web Server

- Check the IP NAT translation.

R1#show ip nat translations

- Result:

- PAT (NAT Overloading)

Given a network as shown in the figure below.

- Config IP address to Routers’ interfaces and PCs. Show the results of each step:

- Configure PAT on Router R1 to translate local IP address of Mail Server.

- Configure default route on router R1:

R1(config)#ip route 0.0.0.0 0.0.0.0 200.20.10.3

- Can Mail Server ping to Web Server (using Command Prompt)? Result:

Yes, Mail Server can ping to Web Server

- Check the IP NAT translation.

R1#show ip nat translations

- Result:

II.4. FileZilla Configuration

- Download and install FileZilla Server and FileZilla Client:

- Download FileZilla Server:

https://filezilla-project.org/download.php?platform=win64&type=server

- Download FileZilla Client:

https://filezilla-project.org/download.php?platform=win64

- Install and Configure FileZilla Server:

- Accessing to FileZilla Server using FileZilla Client:

- Your result:

II.5. Wireshark

a) Wireshark Installation

- Follow the instructions to install Wireshark at:  https://www.wireshark.org/

b) Using Wireshark

- To start to capture packets: At the main window, double-click on the Interface you want to capture the network traffic.

c) Wireshark Practice

- Start up the Wireshark packet sniffer

- Enter the following URL into your browser: http://gaia.cs.umass.edu/wireshark-labs/protected_pages/HTTP-wireshark-file5.html

OR: http://testphp.vulnweb.com/login.php

- Input the username and password. Then, click Sign in.

- Stop Wireshark packet capture and enter “http” in the display-filter-specification window, so that only captured HTTP messages will be displayed in the packet-listing window.

- Question 1: Find the credential (username, password) in the HTTP message.

Answer:

- Question 2: What is the server’s response (status code and phrase) in response to the initial HTTP GET message from your browser?

Answer:

d) Address Resolution Protocol

- Display the contents of the ARP cache on your computer using the command “arp –a”. What is the result of this command?

- Clear your ARP cache using the command “arp –d *” (make sure to open Command prompt using Administration privillege) and make sure your browser’s cache is cleared of previously downloaded documents. What is the result?

- Start the Wireshark packet sniffer.

- Enter the following URL into your browser: http://gaia.cs.umass.edu/wireshark-labs/HTTP-wireshark-lab-file3.html

- Your browser should again display the rather lengthy US Bill of Rights.

- Stop Wireshark packet capture

- Question 1: What is the hexadecimal value of the source address in the Ethernet frame containing the ARP request message sent out by your computer?

Answer:

- Question 2: What is the hexadecimal value of the destination addresses in the Ethernet frame containing the ARP request message sent out by your computer?

Answer: