- 1. Physical Layer (Layer 1)

- Issue: Loose or disconnected cables, power failure, or damaged network hardware.
Solution:

- Check if the network cables are properly connected

- Restart the router/modem and check if it's powered on.

- Try using a different Ethernet cable or Wi-Fi network.

- 2. Data Link Layer (Layer 2)

- Issue: Network interface card (NIC) malfunction, MAC address filtering.
Solution:

- Ensure that the NIC is enabled in the device manager.

- Restart the network adapter and check for driver updates.

- Disable and re-enable the Wi-Fi or Ethernet connection.

- 3. Network Layer (Layer 3)

- Issue: IP address conflicts, incorrect network configuration, router issues.
Solution:

- Run ipconfig /release and ipconfig /renew (on Windows) to refresh the IP.

- Ensure that the router's DHCP is enabled.

- Restart the router and check for firmware updates.

- 4. Transport Layer (Layer 4)

- Issue: Port blocking, firewall restrictions.
Solution:

- Check firewall and antivirus settings to allow network connections.

- Disable VPNs or proxies that may interfere with connectivity.

VLAN-Create

Switch> enable

Switch# config terminal

Switch(config)# hostname S2

S2(config)# vlan 10

S2(config-vlan)# name library

S2(config-vlan)# exit

Assign VLAN to Ports

S2(config)# interface range fastethernet 0/10 - 17

S2(config-if-range)# switchport mode access

S2(config-if-range)# switchport access vlan 10

S2(config-if-range)# exit

Trunking

S2(config)# interface fastethernet 0/2

S2(config-if)# switchport mode trunk

S2(config-if)# switchport trunk allowed vlan 10,20

S1

S1(config)# interface fastethernet 0/1    # this port connects to Router

S1(config-if)# switchport mode trunk

S1(config-if)# switchport trunk allowed vlan 10,20,30,40

S1(config-if)# exit

S1(config)# interface fastethernet 0/2    # connect to S2

S1(config-if)# switchport mode trunk

S1(config-if)# switchport trunk allowed vlan 10,20

Router ABC

Router> enable

Router# config terminal

Router(config)# hostname ABC

ABC(config)# interface fastethernet 0/1

ABC(config-if)# no shutdown

ABC(config-if)# exit

ABC(config)# interface fastethernet 0/1.10

ABC(config-subif)# encapsulation dot1Q 10

ABC(config-subif)# ip address 192.168.10.1 255.255.255.0

ABC(config-subif)# exit

# Lặp lại tương tự cho 0/1.20, 0/1.30, 0/1.40

DHCP

ABC(config)# ip dhcp pool vlan10

ABC(dhcp-config)# network 192.168.10.0 255.255.255.0

ABC(dhcp-config)# default-router 192.168.10.1

ABC(dhcp-config)# dns-server 203.15.5.6

ABC(dhcp-config)# exit

ABC(config)# ip dhcp excluded-address 192.168.10.1

Routing ABC

ABC(config)# interface serial 2/0

ABC(config-if)# ip address 200.20.10.2 255.255.255.0

ABC(config-if)# no shutdown

RIP

ABC(config)# router rip

ABC(config-router)# version 2

ABC(config-router)# network 192.168.10.0

# Lặp lại cho các mạng:

# 192.168.20.0

# 192.168.30.0

# 192.168.40.0

# 200.20.10.0

NAT

ABC(config)# access-list 1 permit 10.0.0.0 0.255.255.255

ABC(config)# ip nat inside source list 1 interface serial2/0 overload

ABC(config)# interface fastethernet 0/2

ABC(config-if)# ip nat inside

ABC(config)# interface serial 2/0

ABC(config-if)# ip nat outside

ABC(config)# ip route 0.0.0.0 0.0.0.0 200.20.10.3