MẠNG MÁY TÍNH 2024-2025

Quarter 4

Section 1 (60 marks)

Question 1: Which layer of the OSI model does a Switch device typically reside at?

A. Physical layer

B. Data-link layer

C. Network layer

D. None of these

Question 2: Which layer of the OSI model does IP protocol work at?

A. Network layer

B. Datalink layer

C. Physical layer

D. Transport layer

Question 3: Given the binary version of an IP as follow: 10101100.00010000.00000010.10100000

What is the decimal version of this IP?

A. 172.16.0.0|

B. 172.16.2.0

C. 172.0.1.0

D. 172.16.1.0

Question 4: Which one of the following is NOT a network topology?

A. Star

B. Bus

C. Ring

D. Sphere

Question 5: What is the prefix length notation for the subnet mask 255.255.240.0?

A. /20

B. /21

C. /19

D. /18

Question 6: What is the subnet mask of the network 10.10.114.0/12?

A. 255.254.0.0

B. 255.224.0.0

C. 255.252.0.0|

D. 255.240.0.0

Question 7: Which of the following best describes the Client/Server network architecture?

A. A network where all devices are connected directly to each other without a central server.

B. A network where a central server provides resources and services to multiple client devices.

C. A network where each device performs both client and server roles equally.

D. A network that uses only wireless connections to communicate between devices.

Question 8: What is the default subnet mask for Class B IP address?

A. 255.255.0.0

B. 255.255.255.0|

C. 255.0.0.0

D. 255.255.255.255

Question 9: What is port number of HTTPS protocol?

A. 80

B. 221

C. 443

D. 21

Question 10: What is the private IP range for Class B?

A. 172.16.0.0 to 172.31.255.255

B. 10.0.0.0 to 10.0.255.255

C. 192.168.0.0 to 192.168.255.255

D. 0.0.0.0 to 0.0.0.255

Question 11: What is a network protocol?

A. A hardware device used to connect different networks

B. A set of rules and conventions for communication between network devices

C. A software application used for browsing the Internet

D. A type of computer network security measure

Question 12: Which protocol operates on port 53 and is used for translating domain names to IP addresses?

A. DHCP

B. TCP

C. DNS

D. UDP

Question 13: What is the port number for FTP protocol?

A. 20

B. 221

C. 22

D. 21

Question 14: What is the name of the missing layer in the image below?

A. Presentation layer

B. Internet layer

C. Access layer

D. None of these

Question 15: Which device is typically responsible for managing network traffic and providing internet access to other devices?

A. Switch

B. Hub

C. Router

D. All of them

Question 16: What does the word LAN stand for?

A. Local Array Net

C. Local Area Node

B. Local Area Network

D. Local Array Node

Question 17: Which of the following IP addresses belongs to class A?

A. 121.12.12.248

B. 128.12.12.248

C. 129.12.12.248

D. 130.12.12.248

Question 18: What is the public IP range for Class A?

A. 10.0.0.0 to 10.255.255.255

C. 128.0.0.0 to 191.255.0.0

B. 1.0.0.0 to 127.0.0.0

D. 172.16.0.0 to 172.31.255.255

Question 19: What is the command generated the output shown in the figure below?

A. ping

B. route

C. netstat

D. nslookup

Question 20: Assume that we have the network 200.10.0.0/24 and we want to divide it into 4 subnets.

What is the last IP address of the 1st subnet?

A. 200.10.0.126

B. 200.10.0.127

C. 200.10.0.62

D. 200.10.0.63

Section 2 (40 marks)

Question 1: A given network address: 205.11.2.0/24. Divide the above network into 16 subnets and fill in the table below (20 marks)